ab197001 Triose Phosphate Isomerase (TPI) Activity Assay Kit (Colorimetric)

Instructions for Use

For the rapid, sensitive and accurate measurement of Triosephosphate Isomerase activity in a variety of samples.

This product is for research use only and is not intended for diagnostic use.

PLEASE NOTE: With the acquisition of BioVision by Abcam, we have made some changes to component names and packaging to better align with our global standards as we work towards environmental-friendly and efficient growth. You are receiving the same high-quality products as always, with no changes to specifications or protocols.

Table of Contents

INTRODUCTION

1.	BACKGROUND	2

2.	ASSAY SUMMARY	3

GENERAL INFORMATION

3.	PRECAUTIONS	4

4.	STORAGE AND STABILITY	4

5.	MATERIALS SUPPLIED	5

6.	MATERIALS REQUIRED, NOT SUPPLIED	5

7.	LIMITATIONS	6

8.	TECHNICAL HINTS	7

ASSAY PREPARATION

9.	REAGENT PREPARATION	8

10.	STANDARD PREPARATION	9

11.	SAMPLE PREPARATION	10

ASSAY PROCEDURE and DETECTION

12.	ASSAY PROCEDURE and DETECTION	12

DATA ANALYSIS

13.	CALCULATIONS	14

14.	TYPICAL DATA	16

RESOURCES

15.	QUICK ASSAY PROCEDURE	18

16.	TROUBLESHOOTING	19

17.	FAQ	21

18.	INTERFERENCES	22

19.	NOTES	23

## BACKGROUND

Triose Phosphate Isomerase (TPI) Activity Assay Kit (colorimetric) (ab197001) provides a quick and easy way for monitoring triosephosphate isomerase activity in a variety of samples. In this kit, triosephosphate Isomerase converts dihydroxyacetone phosphate into glyceradehyde-3-phosphate, which reacts with the enzyme mix and developer to form a colored product with strong absorbance at 450 nm. The assay is simple, sensitive and high-throughput and can detect triose phosphate isomerase activity as low as 40 mU/mL.

Triose Phosphate Isomerase (TPI) is an important enzyme for glycolysis. It catalyzes the reversible conversion of dihydroxyacetone phosphate to glyceraldehyde-3-phosphate, thus maintaining the equilibrium of these two triose phosphates. TPI connects glycolysis to pentose phosphate pathway and lipid metabolism. It is a stable homodimer found in almost all organisms. In humans, TPI deficiency is a rare multisystem disorder and leads to progressive neurological dysfunction, characterized by hemolytic anemia, cardiomyopathy and progressive neuromuscular impairment.

Enzyme mix

+ Developer

Triose phosphate isomerase

Dihydroxacetone                       Glyceraldehyde-3-                                 Color

phosphate                                     phosphate                                   (OD450 nm)

## ASSAY SUMMARY

Standard curve preparation

Sample preparation

Add reaction mix

Measure optical density (OD450 nm) in a kinetic mode at 37°C 20 – 40 min*

*For kinetic mode detection, incubation time given in this summary is for guidance only.

## PRECAUTIONS

Please read these instructions carefully prior to beginning the assay.

All kit components have been formulated and quality control tested to function successfully as a kit. Modifications to the kit components or procedures may result in loss of performance.

## STORAGE AND STABILITY

Store kit at -20ºC in the dark immediately upon receipt. Kit has a storage time of 1 year from receipt, providing components have not been reconstituted.

Refer to list of materials supplied for storage conditions of individual components. Observe the storage conditions for individual prepared components in section 5.

Aliquot components in working volumes before storing at the recommended temperature. Reconstituted components are stable for 2 months.

## MATERIALS SUPPLIED

| Item | Amount | Storage Condition (Before Preparation) | Storage Condition (After Preparation) |
| --- | --- | --- | --- |
| Assay Buffer II/TPI Assay Buffer | 25 mL | -20°C | 4°C / -20°C |
| TPI Substrate/TPI Substrate (lyophilized) | 1 vial | -20°C | -20°C |
| TPI Enzyme Mix/TPI Enzyme Mix (lyophilized) | 1 vial | -20°C | -80°C |
| Developer Solution III/TPI Developer (lyophilized) | 1 vial | -20°C | -20°C |
| NADH Standard I/NADH Standard (lyophilized) | 1 vial | -20°C | -20°C |
| TPI Positive Control/TPI Positive Control (lyophilized) | 1 vial | -20°C | -80°C |

## MATERIALS REQUIRED, NOT SUPPLIED

These materials are not included in the kit, but will be required to successfully perform this assay:

- MilliQ water or other type of double distilled water (ddH2O)

- PBS

- Microcentrifuge

- Pipettes and pipette tips

- Colorimetric microplate reader – equipped with filter for OD450 nm

- 96 well plate: clear plates for colorimetric assay

- Heat block or water bath

- Dounce homogenizer or pestle (if using tissue)

## LIMITATIONS

- Assay kit intended for research use only. Not for use in diagnostic procedures.

- Do not use kit or components if it has exceeded the expiration date on the kit labels.

- Do not mix or substitute reagents or materials from other kit lots or vendors. Kits are QC tested as a set of components and performance cannot be guaranteed if utilized separately or substituted.

## TECHNICAL HINTS

- This kit is sold based on number of tests. A ‘test’ simply refers to a single assay well. The number of wells that contain sample, control or standard will vary by product. Review the protocol completely to confirm this kit meets your requirements. Please contact our Technical Support staff with any questions.

- Keep enzymes and heat labile components and samples on ice during the assay.

- Make sure all buffers and developing solutions are at room temperature before starting the experiment.

- Avoid cross contamination of samples or reagents by changing tips between sample, standard and reagent additions.

- Avoid foaming or bubbles when mixing or reconstituting components.

- Samples generating values higher than the highest standard should be further diluted in the appropriate sample dilution buffers.

- Ensure plates are properly sealed or covered during incubation steps.

- Ensure complete removal of all solutions and buffers from tubes or plates during wash steps.

- Make sure you have the appropriate type of plate for the detection method of choice.

- Make sure the heat block/water bath and microplate reader are switched on before starting the experiment.

## REAGENT PREPARATION

- Briefly centrifuge small vials at low speed prior to opening.

  - Assay Buffer II/TPI Assay Buffer:

Ready to use as supplied. Equilibrate to room temperature before use. Store at 4°C or -20°C.

  - TPI Substrate:

Reconstitute in 220 µL ddH2O. Pipette up and down to dissolve completely. Aliquot substrate so that you have enough volume to perform the desired number of tests. Store at -20°C. Use within 2 months. Keep on ice while in use.

  - TPI Enzyme Mix:

Reconstitute in 220 µL Assay Buffer II/TPI Assay Buffer. Aliquot enzyme so that you have enough volume to perform the desired number of tests. Store at -80°C. Use within 2 months. Keep on ice while in use.

  - Developer Solution III/TPI Developer:

Reconstitute in 220 µL ddH2O. Pipette up and down to dissolve completely. Aliquot developer so that you have enough volume to perform the desired number of tests. Store at -20°C. Use within 2 months. Keep on ice while in use.

  - NADH Standard I/NADH Standard:

Reconstitute the NADH Standard I/NADH Standard in 400 µL of ddH2O to generate a 1.25 mM (1.25 nmol/µL) standard stock solution. Aliquot standard so that you have enough volume to perform the desired number of tests. Store at -20°C. Use within 2 months. Keep on ice while in use.

  - TPI Positive Control:

Reconstitute in 220 µL ddH2O. Aliquot positive control so that you have enough volume to perform the desired number of tests. Store at -70°C. Use within 2 months. Keep on ice while in use.

## STANDARD PREPARATION

- Always prepare a fresh set of standards for every use.

- Diluted standard solution is unstable and must be used within 4 hours.

  - Using the reconstitued 1.25 mM NADH Standard I/NADH standard, prepare standard curve dilution as described in the table in a microplate or microcentrifuge tubes:

| Standard # | Volume of Standard (µL) | Assay Buffer (µL) | Final volume standard in well (µL) | End Conc. NADH in well |
| --- | --- | --- | --- | --- |
| 1 | 0 | 150 | 50 | 0 nmol/well |
| 2 | 6 | 144 | 50 | 2.5 nmol/well |
| 3 | 12 | 138 | 50 | 5.0 nmol/well |
| 4 | 18 | 132 | 50 | 7.5 nmol/well |
| 5 | 24 | 126 | 50 | 10 nmol/well |
| 6 | 30 | 120 | 50 | 12.5 nmol/well |

Each dilution has enough amount of standard to set up duplicate readings (2 x 50 µL).

## SAMPLE PREPARATION

General Sample information:

- We recommend performing several dilutions of your sample to ensure the readings are within the standard value range.

- We recommend that you use fresh samples. If you cannot perform the assay at the same time, we suggest that you complete the Sample Preparation step before storing the samples. Alternatively, if that is not possible, we suggest that you snap freeze cells or tissue in liquid nitrogen upon extraction and store the samples immediately at -80°C. When you are ready to test your samples, thaw them on ice. Be aware however that this might affect the stability of your samples and the readings can be lower than expected.

  - Cell (adherent or suspension) samples:

    - Harvest the amount of cells necessary for each assay (initial recommendation = 1 x 106 cells).

    - Wash cells with cold PBS.

    - Resuspend cells in 100 µL of ice cold Assay Buffer II/TPI Assay Buffer.

    - Homogenize cells quickly by pipetting up and down a few times.

    - Keep on ice for 10 minutes.

    - Centrifuge sample for 5 minutes at 4°C at 10,000 x g using a cold microcentrifuge to remove any insoluble material.

    - Collect supernatant and transfer to a clean tube.

    - Keep on ice.

  - Tissue samples:

    - Harvest the amount of tissue necessary for each assay (initial recommendation = 5 mg).

    - Wash tissue in cold PBS.

    - Resuspend tissue in 100 µL of ice cold Assay Buffer II/TPI Assay Buffer.

    - Homogenize tissue with a Dounce homogenizer sitting on ice, with 10 – 15 passes.

    - Keep on ice for 10 minutes.

    - Centrifuge samples for 5 minutes at 4°C at 10,000 x g using a cold microcentrifuge to remove any insoluble material.

    - Collect supernatant and transfer to a clean tube.

    - Keep on ice.

  - Plasma or Serum:

Serum and plasma samples can be tested directly by adding sample to the microplate wells.

However, to find the optimal values and ensure your readings will fall within the standard values, we recommend performing several dilutions of the sample (1/2 – 1/5 – 1/10).

NOTE: We suggest using different volumes of sample to ensure readings are within the Standard Curve range.

## ASSAY PROCEDURE and DETECTION

●	Equilibrate all materials and prepared reagents to room temperature prior to use.

●	It is recommended to assay all standards, controls and samples in duplicate.

  - Set up Reaction wells:

- Standard wells = 50 µL standard dilutions.

- Sample wells = 2 – 50 µL samples (adjust volume to 50 µL/well with Assay Buffer).

- Positive control = 2 – 20 µL Positive Control (adjust volume to 50 µL/well with Assay Buffer).

- Background control sample wells= 2 – 50 µL samples (adjust volume to 50 µL/well with Assay Buffer). NOTE: for samples that might have high background levels of TPI.

  - Reaction Mix:

Prepare 50 µL of Reaction Mix for each reaction:

| Component | Reaction Mix (µL) | Background Control Reaction Mix (µL) |
| --- | --- | --- |
| Assay Buffer II/TPI Assay Buffer | 44 | 46 |
| TPI Enzyme Mix | 2 | 2 |
| Developer Solution III/TPI Developer | 2 | 2 |
| TPI Substrate | 2 | 0 |

Mix enough reagents for the number of assays (samples, standards, background control and positive control) to be performed. Prepare a master mix of the Reaction Mix to ensure consistency. We recommend the following calculation:

X µL component x (Number samples + Standards + controls +1).

  - Add 50 µL of Reaction Mix into each standard, sample and positive control sample wells.

  - Add 50 µL of background control mix into each background control sample well.

  - Measure absorbance on a microplate reader at OD=450 nm in a kinetic mode, every 2 – 3 minutes, for 20 – 40 minutes at at 37°C.

NOTE: Sample incubation time depends on the TPI activity in the samples. We recommend measuring OD in kinetic mode, and choosing two time points (T1 and T2) in the linear portion of the time course to calculate the TPI activity of the samples.

The NADH Standard Curve can be read in endpoint mode (i.e., at the end of incubation time).

## CALCULATIONS

- Samples producing signals greater than that of the highest standard should be further diluted in appropriate buffer and reanalyzed, then multiplying the concentration found by the appropriate dilution factor.

- For statistical reasons, we recommend each sample should be assayed with a minimum of two replicates (duplicates).

- Average the duplicate reading for each standard and sample.

- If the sample background control is significant, then subtract the sample background control from sample reading.

- Subtract the mean absorbance value of the blank (Standard #1) from all standard and sample readings. This is the corrected absorbance.

- Plot the corrected absorbance values for each standard as a function of the final concentration of TPI.

- Draw the best smooth curve through these points to construct the standard curve. Most plate reader software or Excel can plot these values and curve fit. Calculate the trendline equation based on your standard curve data (use the equation that provides the most accurate fit).

- Extrapolate sample readings from the standard curve plotted using the following equation:

Time point value

=Corrected absorbance-(y-intercept)Slope

Time point = A1 or A2 (and ABG1 or ABG2 for the sample background control)

- Activity of TPI is calculated as:

ΔOD450nm = (A2 – ABG2) – (A1 – ABG1)

- Use the ΔOD to obtain B nmol of NADH generated.

- TPI activity (in nmol/min/mL or mU/mL or U/L) in the test samples is calculated as:

TPI Activity= B(T2-T1)×V*D

= nmol/min/mL = mU/mL

Where:

B = Amount of NADH from Standard Curve (nmol)

T1 = Time of the first reading (A1) in minutes.

T2 = Time of the second reading (A2) in minutes.

V = sample volume added into the reaction well (µL).

D = sample dilution factor.

Sample TPI Activity can also be expressed as mU/mg (nmoles/min NADH generated per mg of protein).

Unit Definition:

1 Unit TPI activity = amount of Triose Phosphate Isomerase that will generate 1.0 µmol of NADH per minute at pH7.4 at 37°C.

## TYPICAL DATA

TYPICAL STANDARD CURVE – Data provided for demonstration purposes only. A new standard curve must be generated for each assay performed.

Figure 1. Typical NADH standard calibration curve using colorimetric reading.

Figure 2: Triose Phosphate Isomerase (TPI) activity detected in a variety of samples.

Figure 3: Relative TPI activity was calculated in lysates prepared from rat muscle (0.5 µg), MCF-7 cells (0.8 µg), and HeLa cells (0.84 µg).

## QUICK ASSAY PROCEDURE

NOTE: This procedure is provided as a quick reference for experienced users. Follow the detailed procedure when performing the assay for the first time.

- Prepare standard, enzyme mix, developer, substrate and assay buffer; (aliquot if necessary); get equipment ready.

- Prepare standard curve.

- Prepare samples in duplicate (find optimal dilutions to fit standard curve readings).

- Set up plate for standard (50 µL), samples (50 µL) and background wells (50 µL) and positive control (50 µL).

- Prepare TPI Reaction Mix (Number samples + standards + 1).

| Component | Reaction Mix (µL) | Background Control Mix (µL) |
| --- | --- | --- |
| Assay Buffer II/TPI Assay Buffer | 44 | 46 |
| TPI Enzyme Mix | 2 | 2 |
| Developer Solution III/TPI Developer | 2 | 2 |
| TPI Substrate | 2 | 0 |

- Add 50 µL of TPI Reaction Mix to the standard, sample and positive control wells.

- Add 50 µL of Background control mix to the background control sample wells.

- Measure output (A1) on a microplate reader at time (T1), at OD450 nm.

- Incubate at 37°C for 20 – 40 minutes, and read absorbance at OD=450 nm in a kinetic mode.

## TROUBLESHOOTING

| Problem | Cause | Solution |
| --- | --- | --- |
| Assay not working | Use of ice-cold buffer | Buffers must be at room temperature |
|  | Plate read at incorrect wavelength | Check the wavelength and filter settings of instrument |
|  | Use of a different 96-well plate | Colorimetric: Clear plates Fluorometric: black wells/clear bottom plate |
| Sample with erratic readings | Samples not deproteinized (if indicated on protocol) | Use PCA precipitation protocol for deproteinization |
|  | Cells/tissue samples not homogenized completely | Use Dounce homogenizer, increase number of strokes |
|  | Samples used after multiple free/ thaw cycles | Aliquot and freeze samples if needed to use multiple times |
|  | Use of old or inappropriately stored samples | Use fresh samples or store at - 80°C (after snap freeze in liquid nitrogen) till use |
|  | Presence of interfering substance in the sample | Check protocol for interfering substances; deproteinize samples |
| Lower/ Higher readings in samples and Standards | Improperly thawed components | Thaw all components completely and mix gently before use |
|  | Allowing reagents to sit for extended times on ice | Always thaw and prepare fresh reaction mix before use |
|  | Incorrect incubation times or temperatures | Verify correct incubation times and temperatures in protocol |

| Problem | Cause | Solution |
| --- | --- | --- |
| Standard readings do not follow a linear pattern | Pipetting errors in standard or reaction mix | Avoid pipetting small volumes (< 5 µL) and prepare a master mix whenever possible |
|  | Air bubbles formed in well | Pipette gently against the wall of the tubes |
|  | Standard stock is at incorrect concentration | Always refer to dilutions on protocol |
| Unanticipated results | Measured at incorrect wavelength | Check equipment and filter setting |
|  | Samples contain interfering substances | Troubleshoot if it interferes with the kit |
|  | Sample readings above/ below the linear range | Concentrate/ Dilute sample so it is within the linear range |

## FAQ

## INTERFERENCES

These chemicals or biological materials will cause interferences in this assay causing compromised results or complete failure:

- RIPA: contains SDS which can destroy/decrease the activity of the enzyme.

## NOTES

Technical Support

Copyright © 2023 Abcam. All Rights Reserved. The Abcam logo is a registered trademark. All information / detail is correct at time of going to print.

For all technical or commercial enquiries please go to:

www.abcam.com/contactus

www.abcam.cn/contactus (China)

www.abcam.co.jp/contactus (Japan)