Version 4a, Last updated 6 September 2023

# ab235887 # Cell Invasion Assay (Collagen I), 24-well, 8 µm

For the measurement of cell migration in response to stimuli.

This product is for research use only and is not intended for diagnostic use.

PLEASE NOTE: With the acquisition of BioVision by Abcam, we have made some changes to component names and packaging to better align with our global standards as we work towards environmental-friendly and efficient growth. You are receiving the same high-quality products as always, with no changes to specifications or protocols.

Table of Contents

1.	Overview	1

2.	Materials Supplied and Storage	2

3.	Materials Required, Not Supplied	3

4.	General guidelines, precautions, and troubleshooting	4

5.	Reagent Preparation	5

6.	Assay Procedure	6

7.	Data Analysis	10

8.	FAQs / Troubleshooting	11

9.	Typical Data	12

10.	Notes	14

Overview

Cell Invasion Assay (Collagen I), 24-well, 8 µm (ab235887) utilizes a Boyden chamber coated with collagen I, where the cells invade the matrix and then migrate through a semipermeable membrane in the Boyden chamber in response to stimulants or inhibitory compounds. The percent cell invasion can be analyzed directly in a plate reader. Our assay is easy to use, sensitive and adaptable to high-throughput systems.

Prepare cells.

Prior to the assay, starve cells for 18-24 hr in serum-free media.

Set up cell invasion assay containing desired chemoattractant in the bottom chamber. Incubate the Cell Chamber (12 x 8µm)/Cell Invasion Chamber at 37°C in CO2 incubator for 2-48 hrs.

Prepare Standard Curve for each cell type.

Wash Cells.

Add Cell dye and incubate at 37°C in CO2 incubator for 60 minutes.

Disassemble the Cell Chamber (12 x 8µm)/Cell Invasion Chamber, remove the top chamber and read the bottom well at Em/Ex = 485/530 nm.

Materials Supplied and Storage

Store kit at -20°C in the dark immediately on receipt and check below for storage for individual components. Kit can be stored for 1 year from receipt, if components have not been reconstituted.

Components are stable for 6 months after the first thaw.

Aliquot components in working volumes before storing at the recommended temperature.

Avoid repeated freeze-thaws of reagents.

| Item | Quantity | Storage temperature (before prep) | Storage temperature (after prep) |
| --- | --- | --- | --- |
| Wash Buffer I/Wash Buffer | 25 mL | -20°C | -20°C |
| Cell Dissociation Solution II/Cell Dissociation Solution | 6 mL | -20°C | -20°C |
| Control Migration Inducer/Control Invasion Inducer | 300 µL | -20°C | -20°C |
| Cell Dye II/Cell Dye | 1 vial | -20°C | -20°C |
| Cell Chamber (12 x 8µm)/Cell Invasion Chamber | 1 | -20°C | RT |
| Collagen I | 1.2 mL | -20°C | 4°C |

Materials Required, Not Supplied

These materials are not included in the kit, but will be required to successfully perform this assay:

- Fluorescence Plate Reader.

- Cotton Swab.

- Centrifuge to spin 96-well plate.

- 96-well clear bottom white plate.

- 1 M Tris, pH-7.5

General guidelines, precautions, and troubleshooting

Please observe safe laboratory practice and consult the safety datasheet.

For general guidelines, precautions, limitations on the use of our assay kits and general assay troubleshooting tips, particularly for first time users, please consult our guide: www.abcam.com/assaykitguidelines

For typical data produced using the assay, please see the assay kit datasheet on our website.

Reagent Preparation

Briefly centrifuge small vials at low speed prior to opening.

Wash Buffer I/Wash Buffer

- Ready to use as supplied.

- Bring to 37°C before use.

- Stable for six months after the first thaw.

Cell Dissociation Solution II/Cell Dissociation Solution

- Ready to use as supplied.

- Bring to 37°C before use.

- Stable for six months after the first thaw.

Control Migration Inducer/Control Invasion Inducer

- Ready to use as supplied.

- Bring to 37°C before use.

- Stable for six months after the first thaw.

Cell Dye II/Cell Dye

- Add 100 µL of DMSO to the vial.

- Aliquot and store at -20°C.

- Bring to 37°C before use.

Cell Chamber (12 x 8µm)/Cell Invasion Chamber

- Open under sterile conditions.

- Keep at room temperature.

Collagen I

- Aliquot under hood and store at 4ºC, if needed.

Assay Procedure

- Equilibrate all materials and prepared reagents to room temperature just prior to use and gently agitate.

- Assay all standards, controls and samples in duplicate.

Cell Invasion Assay Protocol:

- Add 100 µL of Collagen I to coat desired wells of the Top Chamber. Incubate plate at 37°C in an incubator for 2-3 hours or overnight at 4ºC to form a thin film of Collagen I.

∆Note: Check the chamber from the side to make sure the plates are dried.

- Wash the coated plate three times with 200 µL of 1 M Tris. Aspirate.

- Grow cells of interest in desired media and culture conditions. Grow enough cells to perform a Cell Invasion Assay and a Standard Curve.

- Adherent cells should be cultured to ~80% confluence.

- Prior to the assay, starve cells for 18-24 hours in a serum-free media (0.5% serum can be used, if needed).

- After starvation, harvest the cells and centrifuge at 1,000 x g, for 5 minutes to pellet cells.

- Resuspend cell pellet in Wash Buffer I/Wash Buffer and count the number of cells using hemocytometer or automated cell counter.

- Resuspend cells at 1 x 106 cells/mL in a serum-free media.

- Under sterile conditions, disassemble the Cell Chamber (12 x 8µm)/Cell Invasion Chamber and carefully remove the plate cover and the top chamber (figure 1).

Figure 1. Cell Invasion plate: The cells are added to the Top Chamber and the Control Migration Inducer/Control Invasion Inducer or chemoattractant are added to the Bottom Chamber.

- Bottom Chamber: Add 600 µL of medium per well containing desired chemoattractant to the bottom chamber.

- In control well(s), we recommend omitting the chemoattractant.

- For Positive Control, add 60 µL of Control Migration Inducer/Control Invasion Inducer to 540 µL of medium in the bottom chamber.

- Reassemble the top and bottom chambers while ensuring no air bubbles are trapped between them.

- Top Chamber: Add 200 µL (2.3 x 105 cells) of cell suspension to each well of the top chamber.

- Add desired stimulator or inhibitor to the top well, and gently mix.

- Make up the volume to 100 µL with media. Carefully place the plate cover and incubate the Cell Migration Chamber at 37°C in CO2 incubator for 2-48 hours.

Standard Curve:

- Each cell type requires a separate Standard Curve. Prepare a Standard Curve by adding 50 µL cell suspension (1 x 106 cells/mL, 50,000 cells) per well in a 96-well plate (white plate clear bottom).

- Serially dilute the cells 1:1 in Wash Buffer I/Wash Buffer and generate a Standard Curve of cells (50,000, 25,000, 12,500, 6,250, 3,125, 1,562, 781 and 390) in 100 µL total volume.

- As blank, use 100 µL of Wash Buffer I/Wash Buffer.

- Dilute Cell Dye II/Cell Dye 1:250 in PBS and add 50 μl of diluted Cell Dye II/Cell Dye to each well.

- Incubate at 37°C for 1 hour.

- Read the fluorescence at Ex/Em = 485/530 nm.

- Plot the Standard Curve of Number of Cells Vs RFU obtained.

- Fit the data points using a linear trendline with zero intercept.

- The equation for the straight line and R-square value are used for data analysis of samples.

Note: The Cell Invasion RFU reading should fall in the linear range of the Standard Curve. We recommend using triplicates for Standard Curve.

Data Collection and Analysis:

- After the desired incubation with cell invasion inducers/inhibitors, carefully remove the plate cover and aspirate media from the top chamber without puncturing the membrane and matrix.

- Remove cells from the top chamber using a cotton swab. Disassemble the Cell Chamber (12 x 8µm)/Cell Invasion Chamber by removing the top chamber.

- Invert the top chamber and set it aside. Place the plate cover on top of bottom chamber and centrifuge the plate at 1,000 x g for 5 minutes at room temperature.

- Carefully aspirate the media from the bottom chamber, and wash the chamber with 500 µL Wash Buffer I/Wash Buffer.

- Centrifuge the plate at 1,000 x g for 5 minutes at room temperature and aspirate the media from the bottom chamber.

- Add 50 µL of Cell Dye II/Cell Dye (1/250 diluted) in 450 µL of Cell Dissociation Solution II/Cell Dissociation Solution. Mix well. Make desired amount of Cell Dye II/Cell Dye solution depending on the number of wells.

- Add 500 µL of the mix to each well of the bottom chamber.

- Reassemble the Cell Chamber (12 x 8µm)/Cell Invasion Chamber by placing the top chamber into the bottom chamber. Incubate at 37°C in CO2 incubator for 30 minutes.

- Disassemble the Cell Chamber (12 x 8µm)/Cell Invasion Chamber, remove the top chamber and transfer 100 µL of mix from the bottom chamber to the 96-well white plate (the same plate as the standards).

- Read the bottom well at Em/Ex = 485/530 nm. Multiply the reading by 5 to account for the 5X higher volume in each well of the 24-well plate.

Note: Invasive cells pass through the basement membrane and cling to the outer side of the top chamber. Non-invasive cells stay in the upper chamber.

Note: During incubation with Cell Dissociation Solution II/Cell Dissociation Solution/Cell Invasion Dye, gently tap the plate on the side to ensure optimal dissociation of the invasive cells that cling to the outer side of the top chamber.

Data Analysis

- Calculate the number of cells invaded using the equation of the straight line obtained from Standard Curve.

- Percentage Invasion can be calculated as follows:

% Invasion=B# Cells in Lower Chamber  Total # Cells added to Top Chamber  *100

FAQs / Troubleshooting

General troubleshooting points are found at www.abcam.com/assaykitguidelines.

Typical Data

Data provided for demonstration purposes only.

Figure 2. Standard Curve: HT-1080 cells were harvested, counted and serially diluted to obtain desired cell number. Cells were incubated according to the protocol with Cell Invasion Dye and fluorescence (Em/Ex 485/530) was measured.

Figure 3. Cell Invasion: 3T3-NIH and HT-1080 cells were starved overnight and treated with Control Migration Inducer/Control Invasion Inducer or remain untreated (No Treatment). Treatment with Control Migration Inducer/Control Invasion Inducer demonstrated a significant increase in invasion of HT 1080 cells as compared to 3T3-NIH control cells.

Notes

Technical Support

Copyright © 2023 Abcam. All Rights Reserved. The Abcam logo is a registered trademark. All information / detail is correct at time of going to print.

For all technical or commercial enquiries please go to: www.abcam.com/contactus

www.abcam.cn/contactus (China)

www.abcam.co.jp/contactus (Japan)