ab283378 – HDAC Inhibitor Drug Screening Kit (Fluorometric)

For the screening of HDAC inhibitors.
For research use only - not intended for diagnostic use.

For overview, typical data and additional information please visit: http://www.abcam.com/ab283378

PLEASE NOTE: With the acquisition of BioVision by Abcam, we have made some changes to component names and packaging to better align with our global standards as we work towards environmental-friendly and efficient growth. You are receiving the same high-quality products as always, with no changes to specifications or protocols.

Storage and Stability

On receipt entire assay kit should be stored at -80°C, protected from light. Upon opening. Avoid repeated freeze/thaw for all non-buffer components.

Materials Supplied

| Item | Quantity | Storage Condition |
| --- | --- | --- |
| HDAC Substrate | 500 μL | -80ºC |
| 10X Assay Buffer XXX/10X HDAC Assay Buffer | 1 mL | -80ºC |
| Developer II/Lysine Developer | 1 mL | -80ºC |
| HDAC Inhibitor/HDAC Inhibitor (Trichostatin A, 1 mM) | 10 μL | -80ºC |
| HeLa Nuclear Extract (5 mg/ml) | 200 μL | -80ºC |

Materials Required, Not Supplied

These materials are not included in the kit, but will be required to successfully utilize this assay:

- 96-well plate with flat bottom. White plates are preferred for this assay.

- Fluorescence microplate reader.

Assay Protocol

Before using the kit, spin the tubes prior to opening. Warm 10X Assay Buffer XXX/HDAC Assay Buffer to room temperature (RT) before use.

Screen compounds, Inhibitor Control and Positive Control Preparations:

Dissolve HDAC Inhibitor/candidate inhibitors into proper solvent.

Dilute to 2X the desired test concentration with ddH20.

Add 50 l of diluted HDAC Inhibitor/candidate inhibitor into well(s).

For Positive Control, add 50 l ddH20 only.

For Negative Control, add 48 l of ddH20 and 2 l of Trichostatin A.

Reaction Mix Preparation:

Mix enough reagents for the number of assays to be performed. For each well, prepare 50 µl Reaction Mix containing:

| Item | Volume |
| --- | --- |
| 10X Assay Buffer XXX/10X HDAC Assay Buffer | 10 µL |
| HeLa Nuclear Extract | 2 µL |
| HDAC Substrate | 5 µL |
| ddH20 | 33 µL |

Mix well. Add 50 µl of the Reaction Mix into each well. Mix well. Incubate plate at 37°C for

30 min (or longer if desired).

Stop the reaction by adding 10 µl of Developer II/Lysine Developer and mix well. Incubate the plate at

37°C for 30 min.

Measurement:

Read sample in a fluorescence plate reader with Ex. = 350-380 nm and Em. = 440-460 nm. Signal should be stable for several hours at RT.

Calculations:

Set the RFU of Positive Control as the 100%, and calculate the relative activity remains with candidate compounds as follow.

- Activity Remaining With Candidate Compounds = RFU of candidateRFU of Positive Control  ×100

Technical Support

- Copyright © 2023 Abcam. All Rights Reserved. The Abcam logo is a registered trademark. All information / detail is correct at time of going to print.

- For all technical or commercial enquiries please go to:

- www.abcam.com/contactus

- www.abcam.cn/contactus (China)

- www.abcam.co.jp/contactus (Japan)