ab284511 – Monoamine Oxidase B (MOA-B) Inhibitor Screening Kit (Fluorometric)

For the screening of potential MOA-B inhibitors.
For research use only - not intended for diagnostic use.

For overview, typical data and additional information please visit: http://www.abcam.com/ab284511

PLEASE NOTE: With the acquisition of BioVision by Abcam, we have made some changes to component names and packaging to better align with our global standards as we work towards environmental-friendly and efficient growth. You are receiving the same high-quality products as always, with no changes to specifications or protocols.

Storage and Stability

On receipt entire assay kit should be stored at -20°C, protected from light. Kit has a storage time of 1 year from receipt, providing components have not been reconstituted.

Materials Supplied

| Item | Quantity | Storage Condition |
| --- | --- | --- |
| Assay Buffer XVIII/MAO-B Assay Buffer | 25 mL | -20ºC |
| MAO Substrate/MAO-B Substrate | 1 vial | -20ºC |
| MAO-B Enzyme | 1 vial | -20ºC |
| MAO-B Inhibitor/Inhibitor Control (Selegiline) | 1 vial | -20ºC |
| OxiRed Probe (in DMSO) | 0.2 mL | -20ºC |
| Developer Solution V/Developer | 1 vial | -20ºC |

Materials Required, Not Supplied

- 96-well black plate with flat bottom

- Multi-well spectrophotometer

Reagent Preparation

- Before using the kit, spin the tubes prior to opening.

Assay Buffer XVIII/MAO-B Assay Buffer: Bring to room temperature before use. Store at -20°C.

OxiRed Probe: Bring to room temperature before use. Protect from light & moisture. Store at -20°C. Stable for two months.

MAO-B Enzyme: Reconstitute with 25 µL Assay Buffer XVIII/MAO-B Assay Buffer. Mix well. Aliquot & store at -80°C. Stable for two months.

MAO Substrate/MAO-B Substrate: Reconstitute with 110 µL ddH2O. Store at -20°C. Stable for two months.

Developer Solution V/Developer: Reconstitute with 220 µL Assay Buffer XVIII/MAO-B Assay Buffer. Mix well. Store at -20°C. Stable for two months.

MAO-B Inhibitor/Inhibitor Control (Selegiline): Reconstitute with 250 µL ddH2O to make a stock solution of 2 mM. Mix well. Make a 10 µM working solution by adding 5 µL of the 2 mM stock solution into 995 µL ddH2O. Store the stock solution at -20°C. Stable for two months. Inhibitor’s working solution can be stored at 4°C to use within 24 hrs.

Assay Protocol

Screening Compounds, Inhibitor Control, and Blank Control Preparations

- Dissolve candidate test compounds into proper solvent.

- Dilute to 10X concentration with Assay Buffer XVIII/Assay Buffer.

- Add 10 µL diluted test compounds (S), working solution of Inhibitor Control (IC) and Assay Buffer XVIII/MAO-B Assay Buffer (Enzyme Control; EC) into assigned wells.

Δ Note: Preferred final solvent concentration should not be more than 2% by volume. If solvent exceeds 2% include a Solvent Control to test the effect of the solvent on enzyme activity.

Δ Note: Optional: To check the possible inhibitory effect of test inhibitors on Developer Solution V/Developer, prepare a parallel test inhibitor well (TI). MAO-B Inhibitor/Inhibitor Control-Selegiline does not inhibit the Developer Solution V/Developer.

MAO-B Enzyme Solution preparation:

- Dilute the Enzyme stock solution 5 times by adding 2 µL of MAO-B Enzyme Stock Solution into 8 µL of Assay Buffer XVIII/MAO-B Assay Buffer.

- For each well, prepare 50 µL MAO-B Enzyme Solution:

|  | Enzyme Solution |
| --- | --- |
| Assay Buffer XVIII/MAO-B Assay Buffer | 49 µL |
| Diluted MAO-B Enzyme | 1 µL |

- Mix. Add 50 µL/well into wells containing test inhibitors, Inhibitor Control, & Enzyme Control. Incubate for 10 minutes at 37°C.

Δ Note: Always freshly prepare MAO-B Enzyme working solution. Don’t store the enzyme working solution.

Δ Note: To check the possible inhibitory effect of test inhibitors on Developer Solution V/Developer, replace the 1 µL of diluted MAO-B Enzyme with 1 µL of 10 mM H2O2. Mix & add 50 µL to the TI well. Incubate for 10 minutes at 37°C.

MAO Substrate/MAO-B Substrate Solution Preparation:

- For each well, prepare 40 µL of MAO Substrate/MAO-B Substrate Solution:

|  | Substrate Solution |
| --- | --- |
| Assay Buffer XVIII/MAO-B Assay Buffer | 37 µL |
| MAO Substrate/MAO-B Substrate | 1 µL |
| Developer Solution V/Developer | 1 µL |
| OxiRed Probe | 1 µL |

- Mix well and add 40 µL of the MAO Substrate/MAO-B Substrate Solution into each well. Mix well.

Measurement

Measure the fluorescence (Ex/Em = 535/587 nm) kinetically at 37°C for 10-40 min. Choose two points (T1 and T2) in the linear range of the plot and obtain the corresponding fluorescence values (RFU1 and RFU2).

Calculation

- Calculate the slope for all samples, including Enzyme Control (EC), by dividing the net ΔRFU (RFU2 – RFU1) values by the time Δt (T2 – T1). Calculate % Relative Inhibition as follows:

% Relative Inhibition = (Slope of EC-Slope of S)Slope of EC ×100

Technical Support

Copyright © 2023 Abcam. All Rights Reserved. The Abcam logo is a registered trademark. All information / detail is correct at time of going to print.

For all technical or commercial enquiries please go to:

www.abcam.com/contactus

www.abcam.cn/contactus (China)

www.abcam.co.jp/contactus (Japan)