ab288149 – EZ-Desalt Spin Desalting Columns

For quickly desalting, buffer-exchanging, or cleaning up protein samples in solution.
For research use only - not intended for diagnostic use.

For overview, typical data and additional information please visit: http://www.abcam.com/ab288149

Storage and Stability

Store at 4°C. Do not Freeze

Materials Required, Not Supplied

These materials are not included in the kit, but will be required to successfully utilize this assay:

- Exchange Buffer: If samples need to be in a buffer other than Spin Desalting Column storage buffer (PBS with 0.1% sodium azide), the column has to be equilibrated with the desired buffer, as described in the procedure below, before loading your sample.

- Microcentrifuge tubes: Regular 1.5 ml and 2.0 ml microcentrifuge tubes.

- A centrifuge with a fixed angle or a swing bucket rotor: for 1.5 ml or 2.0 ml microcentrifuge tubes.

Procedure

Column Preparation:

- Snap off the bottom closure and slightly loosen the cap.

- Place the column in a 2.0 ml microcentrifuge tube and centrifuge at 1,500 x g for 1 min. to remove the storage buffer.

Δ Note: With a fixed angle rotor, the resin tends to slant upward toward the centre of rotor; mark that side of microcentrifuge tube.

- Carefully apply 350 µl of desired buffer to the centre of column, place column back to centrifuge tube with the marked side facing outward. Centrifuge for 1,500 x g for 1 min., and remove buffer.

- Repeat step 3 three times, discarding the buffer in centrifuge tubes each time. The column is now ready for sample application.

Sample Application:

- Place the column in a new centrifuge tube (1.5 or 2.0 ml), after making sure that there is no residual solution at the column bottom (wipe it, if any).

- Carefully apply sample (30 to 130 µl) to the centre of the column, and centrifuge at 1,500 x g for 2 min.

- Collect purified protein solution in the centrifuge tube.

Technical Support

Copyright © 2021 Abcam. All Rights Reserved. The Abcam logo is a registered trademark. All information / detail is correct at time of going to print.

For all technical or commercial enquiries please go to:

www.abcam.com/contactus

www.abcam.cn/contactus (China)

www.abcam.co.jp/contactus (Japan)