ab289834 – Mycoplasma PCR Detection Kit

PCR based kit to detect mycoplasma in cell culture media. For research use only - not intended for diagnostic use.

For overview, typical data and additional information please visit: http://www.abcam.com/ab289834

Storage and Stability

Upon arrival, the kit should be stored at -20°C. Avoid repeated freeze-thaw cycles of the MasterMix to retain maximum performance. The kit reagents are stable for 12 months if stored properly.

Materials supplied

| Item | Quantity | Storage condition |
| --- | --- | --- |
| ExpressTaq 2X PCR MasterMix | 1.25 mL | -20⁰C |
| Mycoplasma PCR Primer Mix | 100 µL | -20⁰C |
| Mycoplasma Positive Control | 250 µL | -20⁰C |
| Nuclease-free water | 1 mL | -20⁰C |

Materials Required, Not Supplied

These materials are not included in the kit, but will be required to successfully utilize this assay:

- Thermal Cycler

- PCR tubes

- Nuclease-free water

- 1.5 ml microcentrifuge tubes

- Reagents and equipment for agarose gel electrophoresis

- Ethidium bromide or Safe Image™ Basic DNA Stain

- DNA ladder

PCR Detection Protocol

- Bring all reagents and samples to room temperature 30 minutes prior to the assay.

- It is recommended that all standards and samples be run at least in duplicate.

- The cells should remain in culture for at least 48-72 hours undisturbed prior to screening for the presence of mycoplasmas and should be at least 80% confluent. Collect 2.5 µL directly from culture.

- Mix individual components before use and assemble reaction on ice.

| Components | Test Samples | Positive Control | Negative Control |
| --- | --- | --- | --- |
| ExpressTaq 2X PCR MasterMix | 12.5 µL | 12.5 µL | 12.5 µL |
| Mycoplasma PCR Primer Mix | 1 μL | 1 μL | 1 μL |
| Test sample | 2.5 μL | - | - |
| Mycoplasma Positive Control | - | 2.5 μL | - |
| Nuclease-free water | 9 μL | 9 μL | 11.5 μL |
| Final volume per reaction | 25 µL | 25 µL | - L |

- Gently mix the reaction and briefly centrifuge. Run thermocycling conditions for standard PCR.

- Perform 30-40 cycles of PCR as follows:

| Step | Temperature | Duration | Cycle(s) |
| --- | --- | --- | --- |
| Enzyme Activation | 95⁰C | 3 min | 1 |
| Denaturation | 95⁰C | 15 sec | 30-40 |
| Annealing | 55⁰C | 15 sec | 30-40 |
| Extension | 72⁰C | 15 sec | 30-40 |
| Final extension | 72⁰C | 1 min | 1 |
| Holding | 4⁰C | - | - |

- After PCR, maintain the reaction at 4 °C or store at -20 °C until use.

- Analyze the amplification products by agarose gel electrophoresis.

- Visualize by Safe Image™ Basic DNA Stain or ethidium bromide staining.

- The presence of PCR product of ~500 bp in length indicates that the cell culture tested is contaminated with mycoplasma. Note that the length of the PCR product will vary between 370-550 bp depending on different mycoplasma species/strains.

Recommendations for Optimal Results:

Always keep the reaction mixture chilled on ice prior to running the PCR and start the PCR as soon as the reaction mixture is prepared.

Technical Support

Copyright © 2022 Abcam, All Rights Reserved. The Abcam logo is a registered trademark. All information / detail is correct at time of going to print.

For all technical or commercial enquiries please go to:

www.abcam.com/contactus

www.abcam.cn/contactus (China)

www.abcam.co.jp/contactus (Japan)